How to Convert X’pert Pole Figure Files into POPLA Format

(And How to Make PHILCONV Work on Windows XP)

Jason Wolf, Samuel Lim

January 2007

When a pole figure is acquired from an X’Pert Pro diffractometer the resulting data file has the .xrdml file extension.  xrdml is a Panalytical format that can only be used by Panalytical software such as X’Pert Texture.  (Panalytical manufactures the X’Pert and was formerly called Philips Analytical.  Some people still refer to X’Pert files as Philips files.)  These instructions show how to convert .xrdml files into .raw files which can be used in the POPLA texture analysis software.  The POPLA program is easily found on the internet.

Part 1: Convert .xrdml into .rw1

- Open the .xrdml files in the X’Pert Data Viewer program.

- Use the Convert option under the File menu to convert the pole figure data files into .rw1 files.

- The Convert option is also used to convert pole figure background measurements into .cor files.  However, Convert does not work if pole figure files and background files are open at the same time.

Part 2: How to make the PHILCONV program work on Windows XP

PHILCONV is an old Philips program that converts .rw1 files into POPLA format.

If PHILCONV is working on your computer you may skip this section.

PHILCONV is an MS-DOS program that will not work on the DOS system provided within Windows XP.  PHILCONV is available from Panalytical.

- Download the DOSBox software which is easily found on the internet.

- Create a folder that DOSBox will use as a C: drive.

For example, create the folder whose path is C:\DOSBOX_C

DO NOT USE THE C: DRIVE AS THE DOSBOX C: DRIVE!

- Find the DOSBox file named “dosbox.conf” and open it in Notepad.

- Scroll to the bottom, add the following lines under [autoexec], and save:

mount C "C:\DOSBOX_C"

C:

set temp=c:\temp

- Download PHILCONV and move all of its files into the DOSBOX_C folder.

- Put the .rw1 and .cor files to be converted into a folder with a simple name like PFDATA and move that folder into the DOSBOX_C folder.

- Start the DOSBox program.

- Navigate to the directory containing PHILCONV.EXE and enter PHILCONV

Part 3: Convert .rw1 into .raw

A single .rwl file contains data for one pole figure.

PHILCONV combines several .rwl files and reformats it all into one .raw file.

A .raw file is called a POPLA file and contains all the pole figures for one sample.

POPLA files are text files that can be easily checked and edited.

After starting the PHILCONV program:

- Press any key to continue.

- Select System preparation

Enter the path of the source directory (the folder containing .rw1 files).

Example: C:\PFDATA

Enter the path of the destination directory (the folder for output files).

It can be the same as source directory.

Example: C:\PFDATA

Press Esc

- Select Tex Conversion.

Select 1 Add single pole figures for ODF

Press F5=show names

Highlight a file and press F1=select

Move to the next line (use down arrow).

Repeat with F5, F1, etc. until all files for one sample are entered.

Enter a name on the output file line (8 characters or less).

Press F1=Start Conversion

Press Esc

- Select 4 Tex (RWD) to Popla (RAW)

Highlight an .rwd file and press F1=Convert to POPLA

Press Esc

- Select 5 Tex (COR) to POPLA (DFB)

Highlight a .cor file and press F1=Convert to POPLA

Press Esc

- Press Esc=main menu

- Select X eXit from PC-Conversion

- Enter exit to close DOS or DOSBox.

- The POPLA .raw and .dfb files are now located the PFDATA folder.  If using DOSBox, the PFDATA folder will be found within the DOSBOX_C folder.

Part 4: Fix the .raw text file

Some manual changes must be made to the .raw file.

- Open a .raw file in Notepad.

- The first two lines look something like the following

FLNAME    1/08/07

111    5.0 85.0  5.0360.0 0 1 2 1 3   80    1

Add a space to the front of the second line.

Delete a space before the first 5.0 to maintain the other character positions.

Change the first 0 after 360.0 to a 1

The result should look like the following.

FLNAME    1/08/07

111   5.0 85.0  5.0360.0 1 1 2 1 3   80    1

- Scroll down and make this change for every data set in the .raw file.

- Scroll to the end and place the cursor at the very end of the file.  Delete back to the last digit (do not delete the digit) and press the return key.  The return key is the Enter key with the arrow on it.  It is NOT the Enter key next to the number pad.  Some EOF errors in POPLA are caused by not having a carriage return at the end of the file.

- Save the file.

- Open the .raw file in POPLA.  If you continue to have problems try using two instances of Notepad to directly compare the file with a working .raw file.  Use a font that maintains the same spacing for all characters.  If one character or space is out of position the file may not work.  Look for an example file in the POPLA directories.  If still no luck, try replacing the headers of the problem file with the headers from a working file.  Sometimes hidden characters cause problems.  You could also try pasting a set of intensity data over the data of a working file.  Sometimes there are problems with the space between data sets.

- Don’t worry if the intensity values are lower than the actual values.  POPLA normalizes the data so that the highest intensity value is 9999.  The second to last number in the second line of the .raw file is the scaling factor.  100 means the data has not been normalized.

- To change the orientation convention you will have to alter some of the values in the 1 1 2 1 3 part of the second line.  See the POPLA manual for details.

CONVERSION PROGRAM

SOFTWARE

FOR

CONVERTING FILE-EXTENSIONS

TO

OTHER FILE EXTENSIONS

USER GUIDE

EDITION NOTICE:     First Edition, March 1997

This is the first Edition of this publication and is

intended for use with the following programs:

PC-TEXTURE, PC-MRD, PC-HRS and PC-APD.

TABLE OF CONTENTS

1. INTRODUCTION

1.1 INTRODUCTION CONVERSION PROGRAM

1.2 OVERVIEW OF THE CONVERSION PROGRAM MENU STRUCTURE

1.3 MATRIX OF TEX CONVERSIONS

1.4 MATRIX OF APD, MRD AND HRS-CONVERSIONS

1.5 TOTAL OVERVIEW OF DATA FILES

2. OVERVIEW OF THE PROGRAM

2.1 STARTING CONVERSION PROGRAM

2.2 MAIN MENU

2.3 SYSTEM PREPARATION

2.4 TEX CONVERSION

2.5 MRD CONVERSION

2.6 HRS CONVERSION

2.7 APD CONVERSION

2.8 DOSSHELL

2.9 EXIT FROM PROGRAM

1.   Introduction

1.1  Introduction Conversion Program

The conversion program is a program for convert data

files with a typical file name extension

to data files with another file name extension.

The Conversion-Program is an result of the following

programs:

PC-TEX         (PC-Texture + O.D.F)

PC-MRD         (Materials Research Diffractometer)

PC-HRS         (High Resolution Simulation)

PC-APD         (Automated Powder Diffraction)

Each of these programs has his own conversions.

The problem was when you had one program and you wanted

to do a conversion that was in a

another program, the other program had to be send just

for one conversion.

So it's much easier to put all the conversions of the

four programs above in one conversion

program.

1.2  Overview of the Conversion Program menu structure

Main Menu

|  2.2

|

|

System Preparation-----------------Source directory

|  2.3                     |-----destination directory

|

|

TEX Conversion---------------------Add single pole

figures to one ODF           |-----Texture to ASCII format

|  2.4                     |-----ASCII to Texture format

|                          |-----Texture to POPLA format

|                          |-----Texture to POPLA format

|                          |-----POPLA to Texture format

|                          |-----Texture to ASCII for scan files

|

|

MRD Conversion---------------------MRD to HRS

|  2.5                     |-----HRS to MRD

|                          |-----MRD to ASCII

|                          |-----ASCII to MRD

|                          |-----MRD to APD

|                          |-----APD to MRD

|

|

HRS Conversion---------------------.Cnn or .Inn to .Xnn

|  2.6

|

|

APD Conversion---------------------raw data files to UDF ASCII files

|  2.7                      |----DI files to UDI ASCII files

|                           |----UDF files to raw data (RD) format

|

DosShell

|  2.8

|

Exit Program

2.9

1.3  Matrix of TEX Conversions

C R  C R  R C  R  C D  S  C   S O  S  S

TEX    O W  O W  W O  A  O F  O  H   H D  C  C

1 1  D D  A A  W  R B  D  D   D F  N  A

1 CO1        X      X

2 RW1          X  X

3 COD                                   X

4 RWD                  X

5 RWA     X

6 COA   X

7 RAW

8 COR                       X

9 DFB

10 SOD                                  X

11 CHD                                  X

12 SHD                                  X

13 ODF

14 SCN                                        X

15 SCA

A right conversion is from an extension of the left side

to an extension on the top side.

Example:

from the left side 1. CO1 to COD on the top side.

So the conversion is :   filename.CO1 -> filename.COD

All conversions of TEX are unique, so you can only use

this file-extensions in the TEX-conversion.

1.4  Matrix of APD, MRD and HRS-Conversions

R S  D  O  O D  U U  D  X  O  R  A  Y X  C  I

Fro          D D  e  D  S I  D D  n  n  D  D  n  n n  n  n

m                 r     D    F I  n  n        n  n n  n  n

*2 * *1 *3 * *  * *  *5 *4 *3 *2 *  * *4 *  *

APD

1 RD*2                       X

2 SD*                        X

3 Derived*1                  X

4 OD*3                       X

5 OSD*                       X

6 DI*                          X

7 UDF*       X

8 UDI*

MRD

9  Dnn*5                             X  X  X

10 Xnn*4                          X

11 OD*3                           X

12 RD*2                           X

13 Ann*                                          X

14 Ynn*                                       X

HRS

15 Xnn*4

16 Cnn*                                            X

17 Inn*                                            X

*    = all fileconversions who are unique.

*1   = A2, 2D, PK and FPn.

*2   = These Files correspond with each other if the

scans are of the type 2THETA or 2THETA/OMEGA.

*3   = These Files correspond with each other if the

scans are of the type OMEGA or OMEGA/2THETA.

*4   = These Files correspond with each other if the

scans are of the ASCII format used in HRS.

*5   = May only convert to an .OD or .RD file if it is an

original MRD-file.

A right conversion is from an extension of the left side

to an extension on the top side.

Example:

From the left side 15 Inn to Xnn on the

top side.

So the conversion is:  Filename.Inn -> filename.Xnn

I put MRD, APD and HRS in one matrix, because some extensions of these

programs correspond with each other ( see *2, *3 and *4). The matrix is

build up from the most important extensions (programs) to less important

extensions (programs).

1.5  Total Overview of Data Files

.RW1                     Raw pole figure data

.RWD                     Raw pole figure data

.CO1                     Corrected pole figure data

.COD                     Corrected pole figure data

.NOD                     Normalised pole figures

.CAD                     Calculated pole figure data

.COR                     File with correction data

.ODF                     3-dimensional Phi1-Phi-Phi2 ODF data

.INF                     Additional information about the ODF calculation

.SCN                     Scan data

.SCA                     Scan data in ASCII format

.RWA                     Raw pole figure data in ASCII

.COA                     Corrected pole figure data in ASCII

.RAW                     Raw pole figure data in POPLA format

.DFB                     Correction data in POPLA format

.COD .SOD .CHD .SHD      Popla files that can be converted to TEX.ODF files

.Dnn                     Single scans  (internal binary format)

.Ann                     Area scans    (internal binary format)

.Xnn                     Single scans (HRD & HRS format, ASCII)

.Ynn                     Area scans (ASCII)

.RD .OD                  Single Omega scans in PC-APD format (DOS)

.OSD                     Single Omega scans in PC-APD format (WINDOWS)

.Snn                     Sample layer information

.Cnn                     Sample reflectivity profile

.Inn                     Simulation (instrumental convolution)

.RD                      raw data file format (2THETA SCAN DOS)

.SD                      raw data file format (2THETA SCAN WINDOWS)

.A2                      raw data file format

.PK                      raw data file format

.2D                      raw data file format

.FPx                     raw data file format

.SD                      raw data file format

.DI                      DI file format

.UDI                     UDI ASCII file

.UDF                     UDF ASCII file

2.   Overview of the Program

2.1  Starting Conversion Program

- Switch on your Personal Computer (PC).

- Change to your Conversion Program work directory using

the dos CD\(user) command.

- Enter PHILCONV at the DOS prompt.

- Press any key to continue.

The Main menu is displayed.

2.2  Main Menu

Here you can make your choice what you want to do.

It works as follows:

- the selected option is highlighted.

- you can select an option in the following ways:

- Select the option with cursor control(arrow) keys

an then press Enter or F1.

- Enter the capital letter next to the selection you

want.

- Click on the left mouse-button on the option you

want.

From the main menu, depending on which option you select

you will see either:

- Another selection menu which allow you to select a further option.

Or

- An input form. (system preparation menu)

2.3  System Preparation

the Source directory is the directory where all data

files are stored.

the Destination directory is the directory where all data

files should be stored after a conver-

sion.

To change a directory:

- the selected input form is highlighted.

- you can select an input form in the following ways:

- Select the option with cursor control(arrow) keys.

- Click on the left mouse-button on the option you want.

- when you have selected an input form:

- type in the new directory followed by Enter or an

cursor control(arrow) key.

NOTE:     when you use the conversion program for the

first time it's necessary to change the

source- and destination directory to your own

directory settings.

2.4  TEX Conversion

Here you can do a conversion of TEX.

The following conversions are available:

1 Add single pole figure files (.RW1,.CO1) to one (.RWD,.COD) for

ODF calculation.

2 Conversion of TEXTURE -> ASCII format (1 pole figure per file)

Raw data pole figure  xxx.RW1 to xxx.RWA

Corrected pole figure xxx.CO1 to xxx.COA

3 Conversion of ASCII -> TEXTURE format (1 pole figure per file)

Raw data pole figure  xxx.RWA to xxx.RW1

Corrected pole figure xxx.COA to xxx.CO1

4 Conversion of TEXTURE -> POPLA format (3 to 5 pole figures per file)

Raw data pole figure  xxx.RWD to xxx.RAW

5 Conversion of TEXTURE -> POPLA format (1 to 8 correction sets)

Background & defocusing corrections xxx.COR to xxx.DFB

6 Conversion of POPLA -> TEXTURE format (ODF data)

Popla .SOD .COD .SHD .CHD files to xxx.ODF

7 Conversion of TEXTURE -> ASCII format for scan files

Scan data file xxx.SCN to xxx.SCA

Restrictions:

- By conversion 1 you must give at least 3 input files

and the name of the output file before the conversion can be done.

When you have selected an option of above you come in the

file conversion menu of that option.

You can do the following things:

- view a file scan by pressing F5.

- change the source- or destination directory by pressing F9.

- do a file conversion on a file by pressing Enter or F1.

- show the help-menu by pressing F10.

2.5  MRD Conversion

Here you can do a conversion of MRD.

The following conversions are available:

1. From MRD to HRSimulation:

filename.Dnn -> filename.Xnn

Is used to convert MRD .Dnn scan files using scan

axes OMEGA, OMEGA/2THETA, or 2THETA/OMEGA to the format

used for experimental data in HRS.

2THETA/OMEGA scans are converted to OMEGA/2THETA scans.

2. From HRSimulation to MRD:

filename.Xnn -> filename.Dnn

Is used to convert HRS experimental type files to MRD

.Dnn type format.

3. From MRD to ASCII:

All scan types are allowed.

filename.Dnn -> filename.Xnn

filename.Ann -> filename.Ynn

Is used to convert MRD binary scan files to ASCII

format. One dimensional data files with

.Dnn extension become ASCII files with extension

.Xnn. Two dimensional data files with

extension .Ann become ASCII files with extension

.Ynn All scan axes are allowed. The file

headers are similar to those produced by HRS for

expirimental files.

The intensity values are written in counts per

second in the same order in which they are

measured. For .Ynn files the values of the scan axis angles

given in the header correspond to the values for the centre

of the area scan.

4. From ASCII to MRD:

filename.Xnn -> filename.Dnn

filename.Ynn -> filename.Ann

Is used to reserve the action of conversion 3.

5. From MRD to APD:

Omega & Omega/2Theta scans are converted to .OD files

2Theta & 2Theta/Omega scans are converted to .RD files

filename.Dxx -> filenaxx.RD or filenaxx.OD.

Is used to convert MRD scan files (.Dnn extensions)

to the format used in APD. OMEGA and OMEGA/2THETA scans

are converted to .OD files. 2THETA/OMEGA scans are

converted to .RD files. The file named FILENAME.Dnn

becomes FILEAnn.RD or FILEAnn.OD

6. From APD to MRD:

all .OD files are treated as Omega scans.

filename.OD  -> filename.D01

filename.RD  -> filename.D01

Is used to convert APD files to MRD format. All .OD

files are treated as OMEGA scans.

The file named FILENAME.OD becomes FILENAME.D01. The

file named FILENAME.-RD becomes FILENAME.D01.

Restrictions:

- by the file-extensions .Dnn, .Xnn, .Ann and .Ynn -

nn must be a number between 01-99.

- by the conversions 1 and 3 .Dnn may only be converted

to .Xnn if the scan of the .Dnn file is of the type ASCII.

- by conversion 5 are three restrictions:

- a .Dnn file may only be converted to an .OD or .RD

file if the .Dnn file is an original MRD file.

- a .Dnn file may only be converted to an .OD file

if the scan of the .Dnn file is of the type OMEGA or

OMEGA/2THETA.

- a .Dnn file may only be converted to an .RD file

if the scan of the .Dnn file is of the type 2THETA or

2THETA/OMEGA.

The file conversion menu of MRD has the same view and options as

the file conversion menu of TEX (figure 2.5). So further explanation

is not necessary, only that by MRD the files for the chosen conversion

of MRD are shown.

2.6  HRS Conversion

Here you can do a conversion of HRS.

The following conversion is available:

CONVERSION TO .X FILE converts files from PC-HRS .Cnn or

Inn format to ASCII format

with an .Xnn extension. This is equivalent to the ASCII

format used in PC-MRD.

Type of data                            File extension

Sample reflectivity profile               .C01 -.C99

Simulation (instrumental convolution)     .I01 -.I99

Measured scan data from PC-HRD or DCD3.   .X01 -.X99

Restrictions:

By the file-extensions Cnn, Inn and Xnn -

nn must be a number between 01-99

The file conversion menu of HRS has the same view and

options as the file conversion menu of TEX.

So further explanation is not necessary, only that by HRS

the files for the chosen conversion of HRS are shown.

2.7  APD Conversion

Here you can do a conversion of APD.

The following conversions are available:

1. Raw data and calculated 'scan' files can be converted

into UDF ASCII files.

filename.RD    ->   filename.UDF.

filename.A2    ->   filename.UDF.

filename.2D    ->   filename.UDF.

filename.PK    ->   filename.UDF.

filename.SD    ->   filename.UDF.

filename.FPn   ->   filename.UDF.

filename.OD    ->   filename.UDF.

filename.OSD   ->   filename.UDF.

2. DI files can be converted into UDI ASCII files.

filename.DI  ->  filename.UDI.

3. UDF files can be converted into PC-APD raw data file (RD) format.

filename.UDF -> filename.RD.

Restrictions:

by the filename.FPn -

n must be a number between 0-9 or a letter between A-Z.

The file conversion menu of APD is different than those of TEX, MRD

and HRS.

When you have selected an option of above you come in the file conversion

menu of that option.

You can do the following things:

- directly type in a filename to convert

- for a total view of all files that can be converted by this conversion

press F5.

- for changing the source- or destination directory press F9.

2.8  DosShell

The DOS shell option enables you to select and run small

programs without leaving the Conversion program. In addition, all normal

DOS commands can be entered in this option.

- Select DosShell from the main menu.

You can now enter DOS commands and run other programs from the DOS prompt.

You will return to the Conversion program by typing "EXIT".

2.9  Exit from program

You leave here the Conversion program.

All changes in the source- or destination directory will be saved in "gen.syp".

$1.H E L P   I N D E X

F2 = Introduction to PC-CONVERSION

F3 = Use of keys and mouse

F4 = File name conventions

F5 = Main menu items

~2 2 F2=Introduction

~3 3 F3=Keys/mouse

~4 4 F4=Files

~5 10 F5=Main menu

$2.INTRODUCTION to PC-CONVERSION

PC-CONVERSION is a program for convert data files with a typical File name

convention to data files with another File name convention.

For the explanation of the File name convention see Help Index.

NOTE: Before any conversion can be done, you must enter your computer

configuration via: System preparation.

Here you must give your source directory where you have stored the

Data Files en you must give the destination directory where you want

to store the Data File after the conversion.

~10 1 F10=Help index

$3.K E Y B O A R D   A N D   M O U S E

Keyboard control:

Menu screens: The selected item is highlighted (magenta background).

To start a menu item:

- select the item with arrow-up or arrow-down, and

press F1 or Enter;

- directly press the letter in front of the item.

Input forms:  The selected field is highlighted (magenta background).

Select another field with the arrow keys or Enter.

Input forms can have two field types:

Input fields (white on black) can be changed by typing the required

text or number.

A field can be edited by pressing the Insert key.

You can toggle between 'overstrike' and 'insert' mode by

pressing the Insert key again. The current mode is visible

as an O or an I in the lower right corner of the screen.

To recover the old contents press Ctrl & R together.

Function key line: Press Escape or F1 to F10 to start the indicated action.

~10 1 F10=Help index

$3.K E Y B O A R D   A N D   M O U S E

Mouse control:

Pushing the left and right button together is the same as Esc (escape).

The effect of pushing the left or the right button alone depends on the

position of the mouse cursor.

When the cursor is on a:

- Menu item:             go to that item

- Function key field:    start the indicated action

- Non-selected input

or toggle field: select that field

If the mouse cursor is outside any field and you push the left or right

button, the effect is the same as pressing the Enter key. On a:

- Menu screen:           go to the selected item

- Input form:            select the next field

- Prompt field:          accept the default value in the prompt.

~10 1 F10=Help index

$4.FILE NAME CONVENTIONS TEX

Data files in TEX Conversion can have one of the following extensions:

.RW1     Raw pole figure data (spirals/circles, 1 hkl set)

.RWD     Raw pole figure data (spirals/circles, 3 to 5 hkl sets)

.CO1     Corrected pole figure data (spirals/circles, 1 hkl set)

.COD     Corrected pole figure data (spirals/circles, 3 to 5 hkl sets)

.NOD     Normalized pole figures (spirals/circles, 3 to 5 hkl sets)

.CAD     Calculated pole figure data (circles, 3 to 5 hkl sets)

.COR     File with correction data (1 to 8 hkl sets)

.ODF     3-dimensional Phi1-Phi-Phi2 ODF data

.INF     Additional information about the ODF calculation

.SCN     Scan data (counts/s versus 2é, Phi, or Psi)

.SCA     Scan data in ASCII format

.RWA     Raw pole figure data in ASCII (1 hkl set)

.COA     Corrected pole figure data in ASCII (1 hkl set)

.RAW     Raw pole figure data in POPLA format (3 to 5 hkl sets)

.DFB     Correction data in POPLA format (1 to 8 sets)

.COD .SOD .CHD .SHD  Popla files that can be converted to TEX.ODF files.

~10 1 F10=Help index

$4.FILE NAME CONVENTIONS MRD

Data files in MRD Conversion can have one of the following extensions:

.Dnn     Single scans  (internal binary format)

.Ann     Area scans    (internal binary format)

.Xnn     Single scans (HRD & HRS format, ASCII)

.Ynn     Area scans (ASCII)

.RD .OD  Single scans in PC-APD format

nn  can be 01 - 99

~10 1 F10=general help

$4.FILE NAME CONVENTIONS HRS

the system will ask for the name of a .Cnn or .Inn file to be converted

to the format that can be read by PC-HRD.

The following types of Data files in HRS Conversion are used.

They can be recognized by the fileextension, which consists of one

letter and two digits.

Type of data                               File extension

Sample reflectivity profile                .C01 - .C99

Simulation (instrumental convolution)      .I01 - .I99

Measured scan data from PC-HRD or DCD3     .X01 - .X99

~10 1 F10=general help

$4.FILE NAME CONVENTIONS HRS

When you have the option on the function key line:

F2=lower F3=higher ext,

the program will keep the same name and extension letter, but

decrements (or increments) the extension number.

In case of reading the file the numbers of non-existent files

will be skipped; but for writing to the file the next number is taken.

~10 1 F10=general help

$4.FILE NAME CONVENTIONS PC-APD

Data files in PC-APD can have one of the following extensions:

.RD     raw data file format

.A2     raw data file format

.PK     raw data file format

.2D     raw data file format

.FPn    raw data file format

.SD     raw data file format

.DI     DI file format

.UDI    UDI ASCII file

.UDF    UDF ASCII file

~10 1 F10=general help

$10.H E L P   M A I N   M E N U

The following options are available from the main menu:

S  System preparation.

Enter system specific information.

T  TEX Conversion.

File Conversion to and from PC-TEX, POPLA & ASCII format.

M  MRD Conversion.

File conversion to and from PC-HRS, PC-APD & ASCII format.

H  HRS Conversion.

File conversion to .X format.

P  PC-APD Conversion.

File conversion from raw data, DI or UDF files to UDF ASCII, UDI ASCII

& raw data file format.

D  DOS Shell.

The screen will be cleared and  the  DOS  prompt will  be

displayed. Now you may give one or more DOS commands.

To return to the Conversion software, enter DOS command EXIT.

X  Exit from the program.

Any changes in the system parameters are saved in the file 'GEN.SYP'

~10 1 F10=general help

$11.HELP CONVERT FILES TEX

1  Add single pole figure files (.RW1,.CO1) to one (.RWD,.COD) for

ODF calculation.

2  Conversion of TEXTURE -> ASCII format (1 pole figure per file)

Raw data pole figure  xxx.RW1 to xxx.RWA

Corrected pole figure xxx.CO1 to xxx.COA

3  Conversion of ASCII -> TEXTURE format (1 pole figure per file)

Raw data pole figure  xxx.RWA to xxx.RW1

Corrected pole figure xxx.COA to xxx.CO1

4  Conversion of TEXTURE -> POPLA format (3 to 5 pole figures per file)

Raw data pole figure  xxx.RWD to xxx.RAW

5  Conversion of TEXTURE -> POPLA format (1 to 8 correction sets)

Background & defocussing corrections xxx.COR to xxx.DFB

6  Conversion of POPLA -> TEXTURE format (ODF data)

Popla .SOD .COD .SHD .CHD files to xxx.ODF

7  Conversion of TEXTURE -> ASCII format for scan files

Scan data file xxx.SCN to xxx.SCA

NOTE: The directory where the Popla files are stored must be specified in

Main menu - System preparation - Destination Directory.

~10 1 F10=Help index

$12.HELP FILE CONVERSION MRD

The various data directories can be specified via

Main menu - System preparation.

The MRD files are read from, or written to the

source directory.

The HRSimulation files are read from, or written to the

destination directory.

The APD files are read from, or written to the destination directory.

The ASCII files are read from, or written to the destination directory.

Six types of converson:

1. From MRD to HRSimulation:

filename.Dxx -> filename.Xxx.

2Theta/Omega scans are converted to Omega/2Theta scans.

Phi and Psi scans are not allowed.

2. From HRSimulation to MRD:

filename.Xxx -> filename.Dnn.

~10 1 F10=general help

$12.HELP FILE CONVERSION MRD

3. From MRD to ASCII:

All scan types are allowed.

filename.Dnn -> filename.Xnn;

filename.Ann -> filename.Ynn;

see next page for layout of .Ynn files.

4. From ASCII to MRD:

filename.Xnn -> filename.Dnn;

filename.Ynn -> filename.Ann;

5. From MRD to APD:

Omega & Omega/2Theta scans are converted to .OD files;

2Theta & 2Theta/Omega scans are converted to .RD files.

filename.Dxx -> filenaxx.RD or filenaxx.OD.

6. From APD to MRD:

all .OD files are treated as Omega scans.

filename.OD  -> filename.D01;

filename.RD  -> filename.D01;

~10 1 F10=general help

$12.HELP FILE CONVERSION MRD

Layout of ASCII .Ynn files:

The names for the ScanAxis and the AreaAxis can be:

Omega,

2Theta,

Phi,

Psi,

Omega/2Theta (is treated as Omega),

2Theta/Omega (is treated as 2Theta).

The Omega, 2Theta, Phi and Psi values in the heading refer to the

center of the area.

The scan results are written in counts per second, in the same

sequence as they are measured.

The coordinates of the first data point are:

ScanAxis: Center value - (NrOfData-1)/2 * StepWidth;

AreaAxis: Center value - (NrOfScans-1)/2 * AreaStep;

The coordinates of the last data point are:

ScanAxis: Center value + (NrOfData-1)/2 * StepWidth;

AreaAxis: Center value + (NrOfScans-1)/2 * AreaStep;

~10 1 F10=general help

$13.HELP FILE CONVERSION HRS

The source directory and the destination directory can be specified

via Main menu - System preparation.

the system will ask for the name of a .Cnn or .Inn file to be converted

to the format that can be read by PC-HRD.

The following types of Data files in HRS Conversion are used.

They can be recognized by the fileextension, which consists of one

letter and two digits.

Type of data                               File extension

Sample reflectivity profile                .C01 - .C99

Simulation (instrumental convolution)      .I01 - .I99

Measured scan data from PC-HRD or DCD3     .X01 - .X99

~10 1 F10=general help

$13.HELP FILE CONVERSION HRS

When you have the option on the function key line:

F2=lower F3=higher ext,

the program will keep the same name and extension letter, but

decrements (or increments) the extension number.

In case of reading the file the numbers of non-existent files

will be skipped; but for writing to the file the next number is taken.

The data files are read from the source directory and are written to the

destination directory.

~10 1 F10=general help

$14.SYSTEM PREPARATION MENU

Source directory:

Directory where all data files are stored.

Destination directory:

Directory where all data files are stored who will be created,

or the following datafiles:

hpg-files,

apd-files,

ascii-files,

hrs-files and

poplafiles.

~10 1 F10=general help

$15.HELP for UDI/UDF FILE FORMAT

Raw data and calculated 'scan' files can be converted into UDF ASCII files.

DI files can be converted into UDI ASCII files.

UDF files can be converted into PC-APD raw data file (RD) format.

NOTE:

UDI/UDF file format files use the same directory.

This means that UDI/UDF conversion files will be stored in the

same directory where the source-files are stored.

So you only have to change the Source directory to go to the

directory where you have stored your UDI/UDF file format files.

~10 1 F10=general help

$16.HELP COMBINE FILES

Up to 5 input files can be combined to one output file.

The first file name determines the extension: if it is .RW1, the others

must also be .RW1; if it is .CO1, so must be the others.

All files are checked whether they are measured correctly for ODF

calculation:

- Psi-start 0, Psi-step 5, and Psi-end between 70ø and 85ø

- Phi-start 0 and Phi-end 360ø

- Phi-step 6ø for PW1078 and 5ø for the other attachments.

A specified hkl set must be one of the following:

100 110 111 210 211 221 310 311

The first file determines the measurement conditions, the others must

have the same:

- Texture attachment type

- Tube anode and focus

- Generator settings

- Receiving slit height

- Beam width

- Measuring time.

~4 17 F4=hkl indices

~10 1 F10=Help index

$17.NOTE ON CRYSTAL INDICES (hkl)

If the value of a hkl set is requested, the program will convert the

given input combination to its lowest equivalent set (assuming cubic

symmetry) and the result must be one of:

100  110  111  210  211  221  310  311

So:

050 and 009 are converted to 100 and accepted.

306 and 048 are converted to 210 and accepted.

But:

264 will be converted to 321 and will not be accepted.

~10 1 F10=Help index

$*********

! File: GENGB.TXT  LastEditDate: 970307

!

!

!

1.Type a value between ?1 and ?2

2.Type an integer between ?1 and ?2

3.Type a hexadecimal between ?1 and ?2

4.H E L P

5.Esc=quit

6.F1=next page

7.F2=previous page

8.No help available

9.Reading ?1

10.Error while reading ?1

11.OI

12.Toggle argument with + or - key

!

!

!

13.F1=mark corner

14.F9=toggle stepsize

!

! Text used in several modules:

!

15.Esc=main menu

16.Esc=menu

17.Esc=stop

18.Esc=stop batch

!

20.F1=next

21.F1=go to item

22.F1/Enter=accept

23.F1=plot

24.F1=select

25.F1=overwrite

!

28.F2=previous

29.F2=lower

!

31.F3=next page

32.F3=higher ext.

33.F3=delete

!

34.F4,Enter=Accept;

35.F4=insert

36.F5=show names

37.F6=zoom

38.F7=redraw

39.F9=system preparation

!

42.F9=print

43.F9=hardcopy

!

45.F10=help

!

50.NO|YES

51. ... printing ...

52. ... plotting ...

53. ... reading file ?1

54. ... writing file ?1

55.Invalid filename

56.File ?1 not found

57.File ?1 read error

58.File ?1 write error

59.File ?1 opening error

60.File ?1 creation error

61.Invalid directory name

62.Directory not found

63.Disk is write protected

64.No disk in destination drive

65.Disk error in destination drive

66.File ?1 not copied

67.File ?1 not found

68.File ?1 read error

69.Invalid file extension

70.Type a file extension between ?1 and ?2

71.Stay between 01 and 99

72.Instrument off-line

73.Command ?1 gives error ?2

74.Unexpected message: ?1

75.No files for this field

76.No data file specified

77.Existing file: ?1

78.Type 3 integers between -99 and +99

79.HKL set is zero.

80.none

81.Searching

82.Drive ?1 not ready

83.Drive ?1 does not exist

84.Create File ?1

85.Temp-Directory C:\TEMP not found !!!

!

!*****************************************************************************

! Error messages for the prompts for file and directory names

!*****************************************************************************

!

91.Invalid file name

92.File not found

93.Invalid directory name

94.Directory not found

95.Invalid database name

96.Database not found

!

! The number 97 is used in module PDPLOTTR. If you modify it here, then

! change that file too.

!

97.Plot sent to file ?1

!

!*****************************************************************************

! Main Menu

!*****************************************************************************

!

100.M A I N   M E N U

101.STMHPDX

102.S   System preparation

103.T   TEX Conversion

104.M   MRD Conversion

105.H   HRS Conversion

106.P   PC-APD Conversion

107.D   DOS Shell

108.X   eXit from PC-Conversion

!

!*****************************************************************************

! File conversion menu hrs

!*****************************************************************************

!

200.C O N V E R T   F I L E S  H R S

201.C

202.C   Conversion to .X file

!

!*****************************************************************************

! Start up form

!*****************************************************************************

!

210.CONVERSION PROGRAM

211.(9430 927 00831)

212.Version:

!

214.This copy is licenced to:

215.Licence number:

216.System parameters file problem, contact Philips software support

!

!*****************************************************************************

! File conversion menu tex

!*****************************************************************************

!

300.C O N V E R T   F I L E S  T E X

301.1234567

302.1   Add single pole figure files for ODF

303.2   TEX (RW1,CO1) to ASCII (RWA,COA)

304.3   ASCII (RWA,COA) to TEX (RW1,CO1)

305.4   TEX (RWD)  to  POPLA (RAW)

306.5   TEX (COR)  to  POPLA (DFB)

307.6   POPLA (SOD,COD,SHD,CHD) to TEX (ODF)

308.7   TEX (SCN)  to  ASCII (SCA)

!

!*****************************************************************************

! File conversion menu mrd

!*****************************************************************************

!

400.C O N V E R T   F I L E S  M R D

401.123456

402.1   MRD (Dnn) to HRSimulation (Xnn)

403.2   HRSimulation (Xnn) to MRD (Dnn)

404.3   MRD (Dnn,Ann) to ASCII (Xnn,Ynn)

405.4   ASCII (Xnn,Ynn) to MRD (Dnn,Ann)

406.5   MRD (Dnn) to APD (RD,OD)

407.6   APD (RD,OD) to MRD (Dnn)

!

!*****************************************************************************

! File conversion menu pc-apd

!*****************************************************************************

!

420.F I L E   C O N V E R S I O N  P C - A P D

421.SDU

422.S  Scan file  --> UDF

423.D  DI file    --> UDI

424.U  UDF file   --> RD

425.Data file name:

426.Existing files in directory:

427.F5=Show names

428.Converting ?1

429.Existing output file ?1

430.Esc=No conversion

431.F1=Overwrite existing file

432.F2=New name for output file

!

!*****************************************************************************

! System parameters

!*****************************************************************************

!

450.S Y S T E M    P A R A M E T E R S

451.Source directory:

452.Destination directory:

453.From:

454.To:

!

!*****************************************************************************

! Data File Information MRD

!*****************************************************************************

!

500.D A T A   F I L E   I N F O R M A T I O N

501.Type of data:

502.Single scan|Area scan||Reciprocal space scan

503.File:

504.Sample:

505.Measured:

506.Scan-axis

507.Inclination

508.Start      End      Step      Number of steps

509.Converted from PC-APD|Converted from PC-HRD|Converted from ASCII

510.RECEIVING SLIT|TRIPLE AXIS|PARALLEL BEAM

511.Ge(220) two|Ge(440) two|Ge(220) four|Ge(440) four|Ge(220)asym two

512.Cu|Mo|Co|Fe|Cr|Xx

513.LFF|FF|NF|BF

514.PW1710|PW3710|PW3040

515.Sample stage:

516.WAFER-STAGE 1|WAFER-STAGE 2|EULERIAN|EULER + X|PSI-ONLY|HUBER|ATC1|

516.STC|XYZ-STAGE|EULER + XYZ|NO CRADLE

517.hkl:

518.Omega [deg]:

519.2Theta [deg]:

520.Phi [deg]:

521.Psi [deg]:

522.X [mm]:

523.Y [mm]:

524.OMEGA|2THETA|OMEGA-2THETA|2THETA-OMEGA|PHI|PSI|AZIMUTH|REC.SPACE

525.Time per step[s]:

!

!*****************************************************************************

! Batch form hrs

!*****************************************************************************

!

650.B A T C H   P R O C E S S I N G

651.Sample file name:

652.Number of files:

653.Print sample information:

654.Calculate reflectivity profile:

655.Save .C files:

656.Do instrument convolution

657.F1=start batch

658.No sample files specified

659.F1=convert to .X file

!

!*****************************************************************************

! Files form hrs

!*****************************************************************************

!

800.F I L E S

801.Directory:

802.No files available

803.More files than can be displayed

804.To dir:

805.New file name:

806.Existing file

807.F4=copy

808.F5=view

809.F6=rename

810.F7=move

811.F8=delete

812.No view possible for this type of file

!

!*****************************************************************************

! Files form Tex

!*****************************************************************************

!

900.F I L E S

901.Source Directory:

902.No files available

903.More files than can be displayed

904.To dir:

905.New file name:

906.Existing file

907.F4=Copy

908.F5=View

909.F6=Rename

910.F7=Move

911.F8=Delete

912.No view possible for this type of file

!

!*****************************************************************************

! File conversion tex

!*****************************************************************************

!

960.C O M B I N E   P F - F I L E S

961.Input files:

962.Name          hkl

963.Output file:

964.F1=Start conversion

965.Invalid file extension: ?1

966.Invalid Psi-range or step in ?1

967.Invalid Phi-range or step in ?1

968.File ?1, hkl ?2 not allowed in ODF

969.Not enough input files

970.?1 measured with different ?2

971.texture attachment

972.x-ray tube

973.generator setting

974.receiving slit

975.measuring time

976.beam width

!

980.Esc=Previous menu

981.F1=Convert to TEX

982.F1=Convert to ASCII

983.F1=Convert to POPLA

984.Converted from ASCII:

985.Converted from POPLA:

986.Output file name:

987.F2=Ask new name

988.Reflection|Transmission|Merged|Other

989.ATC3|PW1078|ATC3T|OpenCradle|HuberXY|OpenCradleX

990.2Theta/Theta|Phi|Psi|Theta|2Theta

!

!*****************************************************************************

! Files form tex

!*****************************************************************************

!

1000.F I L E S

1001.Source Directory:

1002.No files available

1003.More files than can be displayed

1004.To dir:

1005.New file name:

1006.Existing file

1007.F4=copy

1008.F5=view

1009.F6=rename

1010.F7=move

1011.F8=delete

1012.No view possible for this type of file

;

1020.F I L E    C O N V E R S I O N

1026.F1=convert to APD format

1027.F1=convert to MRD format

1028.F1=convert to HRS format

1029.F1=convert to ASCII format

1030.STEP|CONTINUOUS

1031.Omega|2Theta|Omega/2Theta|2Theta/Omega|Phi|Psi|Azimuth|RecSpace

1032.Invalid file format

1034.No conversion for this type of scan

1035.No original MRD file

!

!*****************************************************************************

! Utilities for directory

!*****************************************************************************

!

1040.Directory name:

1041.Existing directory

1042.Directory created

1043.Cannot create directory

1044.Dir ?1 not empty

1045.F1=erase all files and dir.

1046.Directory removed

1047.Cannot remove directory

1048.Floppy disk drive:

1049.A:|B:

1050.Floppy disk type:

1051.5.25"/360kB|5.25"/1.2MB|3.5"/720kB|3.5"/1.44MB

1052.Ready to erase all data on drive ?1

1053.F1=format floppy disk

1054....formatting a floppy disk...

1055.Give DOS command "EXIT" to return to PC-Conversion

1056.File COMMAND.COM not found

1057.Directory ?1 not found

1058.F1=create

!

!*****************************************************************************

! File information tex

!*****************************************************************************

!

1100.D A T A   F I L E   I N F O R M A T I O N

1101.Type of data:

1102.Raw pole figure|Corrected pole figure|Defoc./Backgr. corrections|

1102.ODF information|ODF data|Calculated pole figure|Normalized pole figure|

1102.Scan data

1103.Wavel:

1104.File:

1105.Sample:

1106.Measured:

1107.Symmetry axis [øPhi]:

1108.Start    End   Step

1109.Psi:

1110.Phi:

1111.Printer not present

1112.Printer error

1113.Pole figure coefficients

1114.C - coefficients

1115.Scan axis      Start      End     Step     Number

1116.Bravais lattice:

1117.FCC|BCC

1118.Rec.slit height [mm]:

1119.Beam width [mm]:

1120.Cu|Mo|Co|Fe|Cr|Xx

1121.LFF|FF|NF|BF|XX

1122.ATC3|PW1078|ATC3-transm.|Open cradle|Huber+XY|Open cradle+X|

1122.STC|Open cradle+XYZ|Unknown

1123.Sample oscillation [mm]:

1124.0|5|10|15|20

1125.0|5|9|15

1126.X[mm]:

1127.Y[mm]:

1128.Z[mm]:

1129.Time per step [s]:

!

1131.hkl:

1132.Defocusing correction:

1133.CORR.FILE|CALCULATED|NONE|UNKNOWN

1134.Correction file name:

1135.Peak width [ø2é]:

1136.Background correction:

1137.CORR.FILE|PRESET VALUES|MEASURED|UNKNOWN

1138.Number of points:

1139.Psi

1140.Background [count/s]:

1141.Second background:

1142.Background angle 1 [ø2é]:

1143.Background angle 2 [ø2é]:

1144.Offset angle 1 [ø2é]:

1145.Offset angle 2 [ø2é]:

1146.Full Phi circles:

1147.Measuring time [s]:

1148.hkl

1149.Peak

1150.2Theta

1151.width

1152.Backg1

1153.Backg2

1154.Scan type:

1155.STEP|CONTINUOUS

1156.Scan axis:

1157.2THETA/THETA|PHI|PSI|THETA|2THETA

!

!*****************************************************************************

! Text used by pc-apd

!*****************************************************************************

!

1200.NO|YES

1201.OFF|ON

1202.FINE|COARSE

1203.Cu|Mo|Co|Fe|Cr|Xx

1204.BF (Broad Focus)|NF (Normal Focus)|FF (Fine Focus)|LFF (Long Fine Focus)

1205.AUTOMATIC|1/30ø|1/12ø|1/6ø|1/4ø|1/2ø|1ø|2ø|4ø

1206.0.05|0.1|0.2|0.3|0.45|0.75|1.2|1.8

1207.GENERAL|STRAIGHT LINE|ADDITION|MATRIX FLUSHING|INTERNAL STANDARD|||

1207.CRYSTALLINITY

1208.ORIGINAL|PURE|ENRICHED

1209.LINEAR|ROOT|LOGARITHMIC

1210.BLACK|BLUE|GREEN|CYAN|RED|MAGENTA|BROWN|WHITE|GRAY|Light BLUE|

1210.Light GREEN|Light CYAN|Light RED|Light MAGENTA|YELLOW|Intense WHITE

1211.PEAK|AREA

1212.LINE|DASHES|POINTS

1213.MEDIUM|COARSE|FINE

1214.UNKNOWN|STANDARD

!

**************