CONVERSION PROGRAM

SOFTWARE

FOR

CONVERSION OF XRD FILES

USER GUIDE

EDITION NOTICE: 	First Edition, May 1997

This is the first Edition of this publication and is intended for use with philconv

TABLE OF CONTENTS

1. Introduction

1.1 Introduction Conversion Program

1.2 Overview of the Conversion Program menu structure

1.3 Overview of Data types and extensions

1.4 Matrix of APD, MRD and HRS-Conversions

1.5 Matrix of TEX Conversions

2. Overview of the Program

2.1 Starting Conversion Program

2.2 Main Menu

2.3 System Preparation

2.4 APD Conversion

2.5 TEX Conversion

2.6 MRD Conversion

2.7 HRS Conversion

2.8 DosShell

2.9 Exit from program

# Introduction

Philips programmes produce data files with specific structure.
In order to be able to use them in other programmes, several programmes contains a conversion
utility.

## Introduction Conversion Program

Several Philips AXR software programmes used for data collection and analysis of X-ray diffrac-
tion (XRD) measurements contain a conversion utility to translate data files from one type of file
structure with each its specific file extension to another.

The problem is that each software program has his own conversions. To do another conversion,
you need another software program.
The solution is to make a Conversion program where all these conversions of several software
packages are available.

The Conversion program PHILCONV is a program for the conversion of XRD data files from one file structure to another file structure.

The Conversion Program is a combination of the conversion utilities of the following programs:

| PC-APD | Automated Powder Diffraction |
| --- | --- |
| PC-HRS | High Resolution Simulation |
| PC-MRD | Materials Research Diffraction |
| PC-TEX | PC-Texture + O.D.F (= Orientation Distribution Function) |

| Note: | Windows 3.1: | When started in a small (windowed) DOS Box, colours will not be displayed. Start the program when the DOS Box is full screen (<ALT> Enter). After that the DOS window may be switched to a smaller window. |
| --- | --- | --- |
|  | Windows95: | Use short directory names (file names). = DOS like (8 characters). |

## Overview of the Conversion Program menu structure

Main Menu

2.2

System Preparation 				Source directory

2.3						Destination directory

APD Conversion				raw data files to UDF ASCII files

2.4						DI files to UDI ASCII files

UDF files to raw data (RD) format

HRS Conversion				Sample Reflectivity Profile files or

2.7						Simulation files to ASCII format

MRD Conversion				MRD to HRS

2.6						HRS to MRD

MRD to ASCII

ASCII to MRD

MRD to APD

APD to MRD

TEX Conversion		 		Add single pole  figures to one ODF

2.5						Texture to ASCII format

ASCII to Texture format

Texture(RWD) to POPLA format(RAW)

Texture(COR) to POPLA format(DFB)

POPLA to Texture format

Texture to ASCII for scan files

DosShell

2.8

Exit Program

2.9

Figure 1.1 Overview of the Conversion Program Menu Structure

## Overview of Data types and extensions

| .2D |  | PC-APD |  | raw data file format |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| .A2 |  | PC-APD |  | raw data file format |  |  |
| .Ann |  | PC-MRD + Epitaxy |  | Area scans (internal binary format) |  |  |
| .CAD |  | PC-TEX |  | Calculated pole figure data |  |  |
| .CHD |  | PC-Texture |  | Popla files that can be converted to TEX.ODF files |  |  |
| .Cnn |  | PC-TEX |  | Sample reflectivity profile |  |  |
| .CO1 |  | PC TEX |  | Corrected pole figure data |  |  |
| .COA |  | PC-TEX |  | Corrected pole figure data in ASCII |  |  |
| .COD |  | PC-TEX |  | Corrected pole figure data |  |  |
| .COD . |  | PC-TEX |  | Popla files that can be converted to TEX.ODF files |  |  |
| .COR |  | PC-TEX |  | File with correction data |  |  |
| .DFB |  | PC-TEX |  | Correction data in POPLA format |  |  |
| .DI |  | PC-APD |  | DI file format |  |  |
| .Dnn |  | PC-MRD + Epitaxy |  | Single scans (internal binary format) |  |  |
| .FPn |  | PROFIT |  | raw data file format |  |  |
| .INF |  | PC-TEX + ODF |  | Additional information about the ODF calculation |  |  |
| .Inn |  | PC-HRS |  | Simulation (instrumental convolution) |  |  |
| .NOD |  | PC-TEX |  | Normalised pole figures |  |  |
| .OD |  | PC-APD |  | Single Omega scans in PC-APD format (DOS) |  |  |
| .ODF |  | PC-TEX |  | 3-dimensional Phi1-Phi-Phi2 ODF data |  |  |
|  | .OSD |  | PC-APD |  | Single Omega scans in PC-APD format (WINDOWS) |  |
| .PK |  | PC-APD |  | raw data file format |  |  |
| .RAW |  | PC-TEX |  | Raw pole figure data in POPLA format |  |  |
| .RD |  | PC-APD |  | raw data file format (2 THETA SCAN DOS) |  |  |
| .RW1 |  | PC-TEX |  | Raw pole figure data |  |  |
| .RWA |  | PC-TEX |  | Raw pole figure data in ASCII |  |  |
| .RWD |  | PC-TEX |  | Raw pole figure data |  |  |
| .SCA |  | PC-TEX |  | Scan data in ASCII format |  |  |
| .SCN |  | PC-TEX |  | Scan data |  |  |
| .SD |  | PC-APD |  | raw data file format (2 THETA SCAN WINDOWS) |  |  |
| .SHD |  | PC-TEX |  | Popla files that can be converted to TEX.ODF files |  |  |
| .Snn |  | PC-HRS |  | Sample layer information |  |  |
| .SOD |  | PC-TEX |  | Popla files that can be converted to TEX.ODF files |  |  |
| .UDF |  | PC-APD |  | UDF ASCII file |  |  |
| .UDI |  | PC-APD |  | UDI ASCII file |  |  |
| .Xnn |  | PC-HRS |  | Single scans (HRD & HRS format, ASCII) |  |  |
| .Ynn |  | PC-MRD |  | Area scans (ASCII) |  |  |

nn= 01. - 99

n = 1. - 9

## Matrix of APD, MRD and HRS-Conversions

| To è From ê | R D | S D | D e r *1 | O D | O S D | D I | U D F | U D I | D n n | X n n | O D | R D | A n n | Y n n | X n n | C n n | I n n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| APD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RD |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| SD |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Derived *1 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| OD |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| OSD |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| DI |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| UDF | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UDI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MRD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dnn *2 |  |  |  |  |  |  |  |  |  | X | X | X |  |  |  |  |  |
| Xnn |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| OD |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| RD |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Ann |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Ynn |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| HRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Xnn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cnn |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| Inn |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |

| *1 | = | A2, 2D, PK and Fpn. (derived from .RD or .SD) |
| --- | --- | --- |
| *2 | = | May only convert to an .OD or .RD file if it is an original MRD-file. |

How to read this matrix:

A conversion is possible from a file type of the left side to a file type on the top side.

Example:

From the left side Inn to Xnn on the top side.

So the conversion is: <filename>.Inn to <filename>.Xnn

## Matrix of TEX Conversions

| To è From ê | C O 1 | R W 1 | C O D | R W D | R W A | C O A | R A W | C O R | D F B | S O D | C H D | S H D | O D F | S C N | S C A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 |  |  | X |  |  | X |  |  |  |  |  |  |  |  |  |
| RW1 |  |  |  | X | X |  |  |  |  |  |  |  |  |  |  |
| COD |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| RWD |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| RWA |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COA | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COR |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| DFB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SOD |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| CHD |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| SHD |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| ODF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCN |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| SCA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to read this matrix:

A conversion is possible from a file type on the left hand side to a file type on the top side.

Example:

from the left side CO1 can be converted to .COD or .COA on the top side.

So the conversion is :   <filename>.CO1 to <filename>.COD

# Overview of the Program

## Starting Conversion Program

- Change to your Conversion Program work directory using the DOS CD\(user) command.

- Enter PHILCONV at the DOS prompt.

The PHILCONV start-up screen is displayed:

Figure 2.1 The Conversion program Start-up Screen

- Press any key to continue.

The Main menu is displayed.

## Main Menu

The Main Menu of the program is shown on the next page.

It is possible to select an option in the following ways:

- Select the option with cursor control (arrow) keys and then press Enter or F1.

- Enter the letter next to the selection you want.

- Click the left mouse-button on the option you want.

Figure 2.2 Main Menu

Depending on the option which you have selected from the main menu, you will see either:

Another selection menu which allow you to select a further option or in case of the System Pre-
paration Menu an input form.

## System Preparation

Figure 2.3 shows the System Preparation Menu.

Figure 2.3 System Preparation Menu

The Source directory is the directory where the data files, which have to be converted are stored.

The Destination directory is the directory where all data files will be stored after a conver-

sion.

To change a directory:

- Select an input field by means of::

- cursor control (arrow) keys.

- or click the left mouse-button on the option you want.

- Type in the new directory followed by Enter or a cursor control (arrow) key.

| Note: | When you use the conversion program for the first time it is necessary to change the source- and destination directory to your own directory settings. |
| --- | --- |

## APD Conversion

Figure 2.8 shows the APD Conversion Menu.

Figure 2.4 APD Conversion Menu

With this function it is possible to perform the conversions as in PC-APD.

The following conversions are available:

- Raw data and calculated 'scan' files can be converted into UDF ASCII files.
	<filename>.RD		to	<filename>.UDF
	<filename>.A2		to	<filename>.UDF
	<filename>.2D		to	<filename>.UDF
	<filename>.PK  	to	<filename>.UDF
	<filename>.SD  	to	<filename>.UDF
	<filename>.FPn 	to	<filename>.UDF
	<filename>.OD  	to	<filename>.UDF
	<filename>.OSD	to	<filename>.UDF

- DI files can be converted into UDI ASCII files.
	<filename>.DI to <filename>.UDI

- UDF files can be converted into PC-APD raw data file (RD) format.
	<filename>.UDF to <filename>.RD

Restrictions:

- For the <filename>.FPn -

n must be a number from 0-9 or a letter from A-Z.

When you have selected an conversion type, it is possible to:

- Type the name of a file to be converted

- Press F5 for a total view of all files that can be converted by this conversion.

- Press F9 for changing the source- or destination directory.

Figure 2.5 File Conversion Menu of an APD Conversion

## TEX Conversion

Figure 2.6 shows the TEX Conversion Menu.

Figure 2.6 TEX Conversion Menu

This function offers the conversions as available in PC-Texture.

The following conversions are available:

- Add single pole figure files (.RW1,.CO1) to one (.RWD,.COD) for ODF calculation.

- Conversion of TEXTURE to ASCII format (1 pole figure per file)

| Raw data pole figure | <filename>.RW1 | to | <filename>.RWA |
| --- | --- | --- | --- |
| Corrected pole figure | <filename>.CO1 | to | <filename>.COA |

- Conversion of ASCII to TEXTURE format (1 pole figure per file)

| Raw data pole figure | <filename>.RWA | to | <filename>.RW1 |
| --- | --- | --- | --- |
| Corrected pole figure | <filename>.COA | to | <filename>.CO1 |

- Conversion of TEXTURE to POPLA format (3 to 5 pole figures per file)

| Raw data pole figure | <filename>.RWD | to | <filename>.RAW |
| --- | --- | --- | --- |

- Conversion of TEXTURE to POPLA format (1 to 8 correction sets)

| Background & defocusing corrections | <filename>.COR | to | <filename>.DFB |
| --- | --- | --- | --- |

- Conversion of POPLA to TEXTURE format (ODF data)

| Popla .SOD .COD .SHD .CHD files | to | <filename>.ODF |
| --- | --- | --- |

- Conversion of TEXTURE to ASCII format for scan files

| Scan data file | <filename>.SCN | to | <filename>.SCA |
| --- | --- | --- | --- |

Restrictions:

- For conversion 1 you must give at least 3 input files (at most 5 input files) and the name of the output file before the conversion can be done.

When you have selected an option above, the file conversion menu of that option will be displayed (figure 2.7).

It is possible to:

- Select a file by means of the arrow keys or the left mouse button and

- convert the file by pressing Enter or F1

- view the file scan by pressing F5.

- Change the source- or destination directory by pressing F9.

- Show the help-menu by pressing F10.

Figure 2.7 Files Conversion Menu of a TEX Conversion

## MRD Conversion

Figure 2.8 shows the MRD Conversion Menu.

Figure 2.8 MRD Conversion Menu

This function offers the conversions as available in PC-MRD.

The following conversions are available:

- From MRD to HRSimulation:
	 <filename>.Dnn 	-> 	<filename>.Xnn

This function converts MRD .Dnn scan files using scan axes OMEGA, OMEGA/2THETA, or 2THETA/OMEGA to the format used for experimental data in HRS.
2THETA/OMEGA scans are converted to OMEGA/2THETA scans.

- From HRSimulation to MRD:
	<filename>.Xnn 	-> 	<filename>.Dnn

This function converts HRS experimental ASCII type files to MRD .Dnn type format.

- From MRD to ASCII:
All scan types are allowed.
	<filename>.Dnn 	-> 	<filename>.Xnn
	<filename>.Ann 	-> 	<filename>.Ynn

This function converts MRD binary scan files to ASCII format.

- One dimensional data files with .Dnn extension become ASCII files with extension .Xnn.

- Two dimensional data files with extension .Ann become ASCII files with extension .Ynn.

- All scan axes are allowed.

- The file headers are similar to those produced by HRS for experimental files.

- The intensity values are written in counts per second in the order in which they are measured.

- For .Ynn files the values of the scan axis angles given in the header correspond to the values for the centre of the area scan.

- From ASCII to MRD:
	<filename>.Xnn 	-> 	<filename>.Dnn
	<filename>.Ynn 	-> 	<filename>.Ann

This function is used to reverse the action of conversion 3.

- From MRD to APD:
Omega & Omega/2Theta scans are converted to .OD files
2Theta & 2Theta/Omega scans are converted to .RD files

	<filename>.Dnn 	to	<filen>Ann.RD or 
					<filen>Ann.OD

This function converts MRD scan files (.Dnn extensions) to the format used in APD.

- OMEGA and OMEGA/2THETA scans are converted to .OD files.

- 2THETA/OMEGA scans are converted to .RD files.

- From APD to MRD:
All .OD files are treated as Omega scans.
	<filename>.OD  	to	<filename>.D01
	<filename>.RD  	to	<filename>.D01

This function converts APD files to MRD format.

- All .OD files are treated as OMEGA scans.

- The file named <FILENAME>.OD becomes <FILENAME>.D01.

- The file named <FILENAME>.RD becomes <FILENAME>.D01.

Restrictions:

- For the file-extensions .Dnn, .Xnn, .Ann and .Ynn 
	nn must be a number between 01-99.

- For the conversions 1 and 3 .Dnn may only be converted to .Xnn if the scan of the .Dnn file is of the type ASCII.

- For conversion 5 are three restrictions:

- A .Dnn file may only be converted to an .OD or .RD file if the .Dnn file is an original MRD file.

- A .Dnn file may only be converted to an .OD file if the scan of the .Dnn file is of the type OMEGA or OMEGA/2THETA.

- A .Dnn file may only be converted to an .RD file if the scan of the .Dnn file is of the type 2THETA or 2THETA/OMEGA.

The file conversion menu of MRD has the same view and options as the file conversion menu of TEX (see figure 2.7).

It is possible to:

- Select a file by means of the arrow keys or the left mouse button and

- convert the file by pressing Enter or F1

- view the file scan by pressing F5.

- Change the source- or destination directory by pressing F9.

- Show the help-menu by pressing F10.

## HRS Conversion

Figure 2.9 shows the HRS Conversion Menu.

figure 2.9 HRS Conversion Menu

This function offers the conversion of HRS.

The following conversion is available:

- Conversion to .X file converts files from PC-HRS .Cnn or .Inn format to ASCII format with an .Xnn extension. This is equivalent to the ASCII format used in PC-MRD.

| Type of data | File extension |
| --- | --- |
| Sample reflectivity profile | .C01 - .C99 |
| Simulation (instrumental convolution) | .I01 - .I99 |
| Measured scan data from PC-HRD or DCD3 | .X01 - .X99 |

Restrictions:

- By the file-extensions Cnn, Inn and Xnn -
	nn must be a number between 01-99

The file conversion menu of HRS has the same view and options as the file conversion menu

of TEX (see figure 2.7).

It is possible to:

- Select a file by means of the arrow keys or the left mouse button and

- convert the file by pressing Enter or F1

- view the file scan by pressing F5.

- Change the source- or destination directory by pressing F9.

- Show the help-menu by pressing F10.

## DosShell

The DOS shell option enables you to select and run programs without leaving the Conversion

program. In addition, all normal DOS commands can be entered in this option.

- Select DosShell from the main menu.

The following screen is displayed.

Figure 2.10 The DOS Shell Screen

You can now enter DOS commands and run other programs from the DOS prompt.

You will return to the Conversion program by typing “EXIT”.

## Exit from Program

Here you leave the Conversion program.

All important changes will be saved in “gen.syp”.